# Central Controller Plan for Haunted House Devices

## Goal

Build a central controller on a Rocky Linux server that can configure, manage, monitor, and manually trigger a mixed fleet of devices: - Arduino 101 nodes - ESP32 nodes - ESP8266 nodes - UNO-based simple I/O nodes where needed

The controller should provide: - A web interface for operators - Device registration and status monitoring - Trigger configuration and live event logging - Manual test controls for relays, scenes, and effects - A simple way to add or replace devices without redesigning the system

## Recommended Architecture

Use a Rocky Linux server as the central application host.

Core components: - Mosquitto for device messaging - FastAPI for the web app and API - PostgreSQL for persistent configuration and logs - Nginx as a reverse proxy and static asset host - systemd or docker compose for service management

Why this approach: - Rocky Linux is stable and suitable for always-on control systems. - MQTT is a good fit for many small devices reporting state and receiving commands. - A web server on Linux is much easier to expand than an embedded controller. - The system can grow from a few boards to dozens without changing the overall pattern.

## High-Level Design

The server is the source of truth for: - Device identity - Trigger mappings - Scene definitions - Operator actions - Event history

Devices are responsible for: - Reporting status and heartbeats - Listening for commands - Firing outputs locally - Sending acknowledgements and errors back to the server

Suggested separation: - Server handles logic, persistence, and UI - Devices handle I/O and local actions - No device should depend on another device being online to keep its own outputs working

## Device Roles

### ESP32

Best all-around network node. Use for: - Relay boards - Sensor boards - LED effects - Wireless trigger nodes - Local display or status devices

### ESP8266

Good low-cost network node. Use for: - Simple relay modules - Basic sensors - Trigger receivers - Budget distributed control points

### Arduino 101

Use as a managed endpoint, not the main hub. Use for: - RF receiver plus relay action nodes - Special effect triggers - Local logic tied to specific radio inputs

### UNO

Use only for simple I/O or legacy devices. Use for: - Basic relay control - Simple serial-connected I/O - Specialized tasks where no networking is required

If an UNO must be networked, pair it with a small network bridge or keep it attached to a network-capable companion.

## Communication Model

Use MQTT as the primary transport.

Why MQTT: - Lightweight for microcontrollers - Easy topic-based routing - Good publish/subscribe model for many devices - Natural fit for status, command, and telemetry messages

Suggested topic structure: - haunt/<device-id>/status - haunt/<device-id>/heartbeat - haunt/<device-id>/event - haunt/<device-id>/trigger - haunt/<device-id>/config - haunt/<device-id>/ack - haunt/<device-id>/error

Optional group topics: - haunt/group/<group-id>/trigger - haunt/scene/<scene-id>/run

## Device Lifecycle

### Registration

Each device should have a unique ID.

On startup it should publish: - Device ID - Firmware version - Hardware type - Capability list - Current online status

### Heartbeat

Each device publishes a heartbeat at a fixed interval. The server uses this to determine online/offline state.

### Commands

The server publishes commands to a device topic. Examples: - Fire relay 1 for 250 ms - Toggle relay 3 - Load configuration block - Enter learning mode

### Acknowledgement

Each device should acknowledge: - Command received - Command completed - Command failed

This is especially useful for haunted-house show control where timing matters.

## Data Model

A PostgreSQL-backed schema is recommended.

Main entities: - Devices - Device capabilities - Groups - Scenes - Triggers - Actions - Event logs - Operator sessions

Suggested tables: - devices - device_capabilities - groups - scenes - scene_steps - triggers - actions - device_configs - event_log - operator_audit

Useful fields for devices: - id - name - type - model - firmware_version - last_seen_at - status - ip_address - notes

Useful fields for triggers: - id - name - source_type - source_value - target_device_id - target_action - cooldown_ms - enabled

## Web UI Scope

The web interface should focus on operator workflow, not configuration complexity.

Core screens: - Dashboard - Device list - Device detail - Trigger editor - Scene editor - Live event log - Manual test panel - Health and diagnostics - Settings and network status

Dashboard should show: - Online/offline counts - Active alerts - Recent trigger activity - Last-seen time for each device - Fast access to manual overrides

Device detail should show: - Device metadata - Capability list - Current config - Recent events - Manual trigger buttons - Last heartbeat and error state

Trigger editor should support: - Mapping one source event to one or more outputs - Delay and cooldown rules - Toggle actions - Momentary pulse actions - Scene membership

## Trigger Logic

The controller should support several trigger types.

### Momentary Pulse

Fire an output for a fixed duration and return it to off. Useful for: - Door strikes - Solenoids - Light flashes - Brief relay activations

### Locked Pulse

Fire once, then block repeat triggers for a configured cooldown. Useful for: - Effects that should not spam - Props that need recovery time - Preventing accidental double-fires

### Toggle

Flip a relay or output on each trigger. Useful for: - Persistent lights - Enabled/disabled states - Latching effects

### Scene Trigger

Run a multi-step sequence with delays between steps. Useful for: - Multi-prop haunted-house scenes - Lighting and sound coordination - Coordinated start/stop behavior

## Suggested Operator Workflow

- Open the web dashboard.

- Check that all devices are online.

- Select a device or scene.

- Assign or verify trigger mappings.

- Test outputs from the manual panel.

- Arm the system for show mode.

- Monitor events and acknowledgements during operation.

- Review logs after the show.

## API Shape

A REST API is a good starting point.

Suggested endpoints: - GET /api/devices - GET /api/devices/{id} - POST /api/devices/{id}/command - POST /api/devices/{id}/config - GET /api/scenes - POST /api/scenes - GET /api/triggers - POST /api/triggers - GET /api/events - GET /api/health

Optional real-time layer: - WebSocket or Server-Sent Events for live status updates

## Configuration Strategy

Keep configuration split into two layers:

### Server-Side Configuration

Stored centrally in PostgreSQL. This includes: - Device names - Trigger mappings - Scene definitions - Operator settings - Global rules

### Device-Side Configuration

Stored locally on the device when needed. This includes: - Device ID - Local fallback trigger settings - Relay timing values - RF code mappings where relevant

This lets a device continue working if the network is temporarily unavailable.

## Security

Even for a local haunted-house network, basic security matters.

Minimum controls: - Put the controller on a private LAN or VLAN - Require login for the web UI - Protect write actions with authentication - Use service accounts for devices - Log operator changes

Optional controls: - HTTPS with a local certificate - MQTT authentication - Role-based access for operators and admins

## Deployment Plan

### Option A: Simple Service Deployment

- Install dependencies directly on Rocky Linux

- Run Mosquitto as a system service

- Run FastAPI with Uvicorn or Gunicorn

- Put Nginx in front of the app

- Use systemd for startup and supervision

### Option B: Container Deployment

- Use Docker or Podman

- Run Mosquitto, API, and database as containers

- Manage the stack with compose files

- Easier to move between servers

For a first version, container deployment is often simpler to reproduce.

## Monitoring

Track: - Device heartbeat age - Trigger success/failure - Command latency - Relay fire counts - Error rates - Offline devices

Useful alerts: - Device missed heartbeat - Scene command failed - MQTT broker unreachable - Database connection lost - Too many repeated trigger events

## Development Phases

### Phase 1: Core Infrastructure

- Set up Rocky Linux host

- Install MQTT broker

- Build the API skeleton

- Create the database schema

- Make one test device report status

### Phase 2: Web UI

- Build dashboard

- Build device list and detail pages

- Add event log view

- Add basic auth

### Phase 3: Trigger Control

- Add trigger editor

- Add manual test actions

- Add scene support

- Add cooldown and toggle rules

### Phase 4: Mixed Device Support

- Add ESP32 device templates

- Add ESP8266 templates

- Add Arduino 101 endpoint support

- Add UNO support where practical

### Phase 5: Operations Hardening

- Add audit logging

- Add backups

- Add offline warnings

- Add restore and recovery steps

## Suggested Repo Layout

central-controller/
  app/
    api/
    auth/
    devices/
    scenes/
    triggers/
    templates/
    ui/
  infra/
    nginx/
    mosquitto/
    systemd/
    docker/
  migrations/
  tests/
  docs/
  scripts/

## Practical Recommendation

For this project, the strongest approach is: - Rocky Linux server as the hub - MQTT for device messaging - FastAPI for the application layer - PostgreSQL for state and logs - Web UI for configuration and monitoring - Simple device firmware on each board type

This gives you a scalable platform that can manage both the Arduino 101 group and the ESP/UNO fleet without forcing every device to run the same code or networking stack.

## Next Build Step

The next useful artifact is a concrete implementation plan with: - exact package list for Rocky Linux - MQTT topic naming rules - database schema SQL - first API endpoints - first web pages - device message format examples

If you want, that can be the next document or I can scaffold the project structure directly.